Your Specialists for electromagnetic Solutions

DC rotary solenoid

Function

⌘ Rotary movement without axial wave movement

⌘ Rotation angle 35°, 65° and 95°

⌘ Increasing torque characteristic

(for smaller duty cycles almost linear torque

characteristic)

⌘ With and without return spring

⌘ Clockwise and anti-clockwise

Construction

⌘ Shaft protruding on both sides

⌘ Spring return force infinitely adjustable

⌘ Armature guided in ball bearings

⌘ Fastening via thread bores at the end faces

⌘ Insulation materials of the excitation winding correspond  to thermal class B

⌘ Electrical connection via free flexible lead ends

⌘ Protection class according to DIN VDE/DIN EN 60529 when properly installed: IP 20

Application examples

Md

⌘ Tooling machines, office machines, packing machines,  textile machines

⌘ Shutter for laser and optical devices

⌘ Measuring and control technology

Options

⌘ Proportional rotary solenoids, double-acting with high

6

Product group

G DA

Fig. 1: G DA with return spring

Adjustment range

of the spring

Md

MRmax.

MRmin.

Rotation angle

torque type GDR

⌘ Reverse and polarized rotary solenoids on request ⌘ Please contact us for application related solutions or  functional units

Standards

⌘ Design and testing according to DIN VDE 0580 ⌘ Quality management to ISO 9001

Magnet-Schultz GmbH & Co. KG • Allgäuer Straße 30 • D-87700 Memmingen • Tel. +49 83311040 • www.magnet-schultz.com

Fig. 2: torque characteristic and return spring characteristic

Technical data G DA X and G DA Y without spring return

| G DA X ... X20 B01 |  | 035 |  |  |  |  | 050 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) |  | 95 +3 |  |  |  |  | 95 +3 |  |  |  |  |
| Operating mode rel. Einschaltdauer |  | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5% |
| (Ncm) 0° Torque Md at 30° 60° 95° |  | 1,68 | 2,00 | 2,15 | 2,20 | 2,00 | 9,00 | 10,00 | 10,20 | 10,00 | 8,40 |
|  |  | 0,73 | 1,45 | 1,75 | 2,00 | 2,45 | 3,90 | 6,55 | 8,10 | 9,50 | 11,00 |
|  |  | 0,34 | 0,80 | 1,20 | 1,45 | 2,00 | 1,50 | 3,25 | 4,80 | 6,60 | 8,90 |
|  |  | 0,22 | 0,58 | 0,90 | 1,30 | 2,25 | 0,77 | 1,70 | 2,55 | 4,10 | 7,40 |
| Rated power P20 (W) |  | 7,0 | 16,2 | 24,2 | 38 | 105 | 13,3 | 27,2 | 41,5 | 75 | 168 |
| Moment of inertia (kgm2) |  | 0,314 x 10-6 |  |  |  |  | 1,903 x 10-6 |  |  |  |  |
| Time constant τ (ms) |  | 6,5 |  |  |  |  | 9,2 |  |  |  |  |
| Solenoid weight mM (kg) |  | 0,16 |  |  |  |  | 0,42 |  |  |  |  |

| G DA Y ... X20 B01 | 025 |  |  |  |  | 035 |  |  |  |  | 050 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 65 +3 |  |  |  |  | 65 +3 |  |  |  |  | 65 +3 |  |  |  |  |
| Operating mode rel. Einschaltdauer | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5% |
| (Ncm) 0° Torque Md at 20° 40° 65° | 0,98 | 1,50 | 1,65 | 1,80 | 1,80 | 2,40 | 2,90 | 3,00 | 3,00 | 3,00 | 13,90 | 15,30 | 15,70 | 15,70 | 14,30 |
|  | 0,46 | 1,00 | 1,30 | 1,55 | 1,85 | 1,50 | 2,35 | 2,75 | 3,10 | 3,60 | 8,80 | 12,90 | 14,60 | 15,80 | 17,10 |
|  | 0,25 | 0,60 | 0,85 | 1,15 | 1,60 | 0,85 | 1,70 | 2,10 | 2,50 | 3,25 | 4,30 | 7,90 | 10,50 | 12,90 | 15,60 |
|  | 0,16 | 0,40 | 0,60 | 0,85 | 1,50 | 0,57 | 1,26 | 1,80 | 2,50 | 3,60 | 2,40 | 5,00 | 7,20 | 10,00 | 14,40 |
| Rated power P20 (W) | 4,2 | 10,9 | 16,6 | 24,3 | 57,6 | 7,0 | 16,2 | 24,2 | 38 | 105 | 13,3 | 27,2 | 41,5 | 75 | 168 |
| Moment of inertia (kgm2) | 1,15 x 10-6 |  |  |  |  | 0,366 x 10-6 |  |  |  |  | 2,04 x 10-6 |  |  |  |  |
| Time constant τ (ms) | 9 |  |  |  |  | 8 |  |  |  |  | 15,5 |  |  |  |  |
| Solenoid weight mM (kg) | 0,07 |  |  |  |  | 0,16 |  |  |  |  | 0,42 |  |  |  |  |

| G DA Y... X20 B03 | 025 |  |  |  |  | 035 |  |  |  |  | 050 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 35 +3 |  |  |  |  | 35 +3 |  |  |  |  | 35 +3 |  |  |  |  |
| Operating mode rel. Einschaltdauer | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5% |
| (Ncm) 0° Torque Md at 10° 20° 35° | 1,05 | 1,60 | 1,90 | 2,00 | 2,10 | 2,50 | 3,30 | 3,60 | 3,90 | 4,30 | 14,40 | 17,70 | 19,20 | 20,00 | 20,40 |
|  | 0,84 | 1,55 | 1,90 | 2,15 | 2,55 | 2,10 | 3,10 | 3,50 | 3,95 | 4,60 | 12,30 | 16,20 | 18,00 | 19,80 | 21,80 |
|  | 0,66 | 1,35 | 1,80 | 2,20 | 2,90 | 1,80 | 2,90 | 3,50 | 4,10 | 5,20 | 10,00 | 15,40 | 18,00 | 20,70 | 24,30 |
|  | 0,38 | 0,90 | 1,30 | 1,70 | 2,60 | 1,20 | 2,40 | 3,00 | 3,70 | 5,10 | 5,90 | 10,60 | 13,60 | 16,90 | 21,70 |
| Rated power P20 (W) | 4,2 | 10,9 | 16,6 | 24,3 | 57,6 | 7,0 | 16,2 | 24,2 | 38 | 105 | 13,3 | 27,2 | 41,5 | 75 | 168 |
| Moment of inertia (kgm2) | 1,40 x 10-6 |  |  |  |  | 0,358 x 10-6 |  |  |  |  | 2,11 x 10-6 |  |  |  |  |
| Time constant τ (ms) | 9 |  |  |  |  | 12 |  |  |  |  | 20 |  |  |  |  |
| Solenoid weight mM (kg) | 0,07 |  |  |  |  | 0,16 |  |  |  |  | 0,42 |  |  |  |  |

Notes on the tables

Due to matural dispersion torque data may deviate by approx.  . ± 10 % from the table values and base on:

• Rated voltage 24 V / 5 %-100 % duty cycle

• 90 % of the rated voltage

• Normal operating temperature at 35° ambient temperature • Mounting on heat-insulating base

| Duty cycle (%) | 100 | 40 | 25 | 15 | 5 |
| --- | --- | --- | --- | --- | --- |
| Operating time (s) | dauernd | 120 | 75 | 45 | 15 |

Rated power P20 applies for a coil temperature of 20 °C 0° is the end position in energized condition.

The torque data Md (Ncm) do not consider any return spring. For  types with return spring, self-aligning torques according to table  „Technical Data with return spring“ are to be considered.

Rated voltage

Rated voltage is 24 V. An adaptation of the exciter coil to a rated  voltage less than 120 V is possible on request. Standard values for voltage and operating mode: 24 V, S1 (100%)  / S3 (40%).

The devices correspond to protection class III. Electrical equipment  of protection class III may be only connected to low voltage  systems (PELV, SELV)(IEC 60364-4-4-41). The design limit  of the equipment is a rated voltage not higher than 120 V (EN  61140:2002) with DC. On request we are pleased to check to what  extent the delivery of higher rated voltages is possible as special  solutions by agreement.

By edition of the present list, all former unit lists lose their validity especially. Illustrations without guarantee – modifications and supply availability reserved

2

Technical data G DA X and G DA Y without return spring

| G DA X ... X20 B01 | 060 |  |  |  |  | 075 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 95 +3 |  |  |  |  | 95 +3 |  |  |  |  |
| Operating mode rel. Einschaltdauer | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % |
| (Ncm) 0° Torque Md at 30° 60° 95° | 19,5 | 22,3 | 22,6 | 22,3 | 17,9 | 34,0 | 33,0 | 31,0 | 28,5 | 19,0 |
|  | 7,5 | 14,2 | 17,0 | 19,6 | 23,4 | 19,4 | 30,0 | 33,5 | 35,5 | 39,0 |
|  | 3,0 | 6,4 | 9,7 | 13,0 | 18,9 | 8,1 | 17,5 | 22,5 | 27,5 | 33,0 |
|  | 1,3 | 3,4 | 5,0 | 7,6 | 14,0 | 4,4 | 11,1 | 16,2 | 20,5 | 35,0 |
| Rated power P20 (W) | 16,8 | 40 | 61 | 96 | 262 | 23,5 | 55 | 82 | 124 | 303 |
| Moment of inertia (kgm2) | 4,88 x 10-6 |  |  |  |  | 14,45 x 10-6 |  |  |  |  |
| Time constant τ (ms) | 18 |  |  |  |  | 25 |  |  |  |  |
| Solenoid weight mM (kg) | 0,74 |  |  |  |  | 1,48 |  |  |  |  |

| G DA Y ... X20 B01 | 060 |  |  |  |  | 075 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 65 +3 |  |  |  |  | 65 +3 |  |  |  |  |
| Operating mode rel. Einschaltdauer | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % |
| (Ncm) 0° Torque Md at 20° 40° 65° | 28,5 | 33,0 | 34,0 | 34,0 | 31,0 | 48,0 | 50,0 | 49,0 | 47,0 | 39,0 |
|  | 15,0 | 25,4 | 29,1 | 31,9 | 35,7 | 37,0 | 48,0 | 53,0 | 55,0 | 58,0 |
|  | 7,9 | 16,8 | 21,6 | 26,0 | 34,6 | 19,0 | 37,0 | 44,0 | 48,0 | 57,0 |
|  | 3,9 | 9,3 | 13,4 | 18,4 | 29,3 | 11,0 | 25,0 | 35,0 | 42,0 | 59,0 |
| Rated power P20 (W) | 16,8 | 40 | 61 | 96 | 262 | 23,5 | 55 | 82 | 124 | 303 |
| Moment of inertia (kgm2) | 5,14 x 10-6 |  |  |  |  | 15,25 x 10-6 |  |  |  |  |
| Time constant τ (ms) | 22,5 |  |  |  |  | 30 |  |  |  |  |
| Solenoid weight mM (kg) | 0,74 |  |  |  |  | 1,48 |  |  |  |  |

| G DA Y... X20 B03 | 060 |  |  |  |  | 075 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 35 +3 |  |  |  |  | 35 +3 |  |  |  |  |
| Operating mode rel. Einschaltdauer | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % | S1 100 % | S3 40 % | S3 25 % | S3 15 % | S3 5 % |
| (Ncm) 0° Torque Md at 10° 20° 35° | 31,0 | 38,0 | 41,0 | 43,0 | 46,0 | 60,0 | 67,0 | 68,0 | 69,0 | 64,0 |
|  | 25,0 | 35,5 | 39,5 | 43,0 | 49,0 | 54,0 | 66,0 | 71,0 | 73,0 | 75,0 |
|  | 18,5 | 32,0 | 38,0 | 43,0 | 53,0 | 47,0 | 65,0 | 73,0 | 79,0 | 88,0 |
|  | 10,8 | 21,5 | 28,0 | 34,5 | 46,0 | 30,0 | 51,0 | 63,0 | 70,0 | 87,0 |
| Rated power P20 (W) | 16,8 | 40 | 61 | 96 | 262 | 23,5 | 55 | 82 | 124 | 303 |
| Moment of inertia (kgm2) | 5,39 x 10-6 |  |  |  |  | 15,92 x 10-6 |  |  |  |  |
| Time constant τ (ms) | 30 |  |  |  |  | 42,5 |  |  |  |  |
| Solenoid weight mM (kg) | 0,74 |  |  |  |  | 1,48 |  |  |  |  |

Return spring - Adjustment

The following table indicates the adjustment range of the spring  return torque depending on the operating mode. (Diagram fig. 3) The spring return torque is to be modified within the adjustment  range by changing the tooth spaces and by turning the spring  housing. For turning the spring housing, the fastening screws are  to be loosened and tightened again.

Md

Md

MRmax.

MRmin.

Adjustment range  of the spring

Rotation angle

Fig. 3: torque characteristic and return spring characteristic

By edition of the present list, all former unit lists lose their validity especially. Illustrations without guarantee – modifications and supply availability reserved

3

Technical data G DA X and G DA Y with return spring

| G DA X ... X20 B21 (clockwise) B25 (anti-clockwise) |  | 035 |  |  |  |  | 050 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) |  | 95 +3 |  |  |  |  | 95 +3 |  |  |  |  |
| Operating mode |  | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5% |
| Self-aligning torque (Ncm) min. of the spring MR max. |  | – – | 0,14 0,18 | 0,14 0,34 | 0,14 0,74 | 0,14 0,74 | 0,3 0,3 | 0,3 0,6 | 0,3 1,25 | 0,3 2,4 | 0,3 3,0 |
| Spring constant (Ncm/°) |  | 0,0065 |  |  |  |  | 0,016 |  |  |  |  |

| G DA Y ... X20 B21 (clockwise) B25 (anti-clockwise) | 025 |  |  |  |  | 035 |  |  |  |  | 050 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 65 +3 |  |  |  |  | 65 +3 |  |  |  |  | 65 +3 |  |  |  |  |
| Operating mode | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5% |
| Self-aligning torque (Ncm) min. of the spring MR max. | 0,1 0,1 | 0,1 0,3 | 0,1 0,4 | 0,1 0,6 | 0,1 0,6 | 0,14 0,27 | 0,14 0,72 | 0,14 1,0 | 0,14 1,4 | 0,14 1,6 | 0,3 0,75 | 0,3 2,16 | 0,3 3,4 | 0,3 3,5 | 0,3 3,5 |
| Spring constant (Ncm/°) | 0,00206 |  |  |  |  | 0,0065 |  |  |  |  | 0,016 |  |  |  |  |

| G DA Y ... X20 B23 (clockwise) B27 (anti-clockwise) | 025 |  |  |  |  | 035 |  |  |  |  | 050 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 35 +3 |  |  |  |  | 35 +3 |  |  |  |  | 35 +3 |  |  |  |  |
| Operating mode | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5% |
| Self-aligning torque (Ncm) min. of the spring MR max. | 0,1 0,3 | 0,1 0,6 | 0,1 0,6 | 0,1 0,6 | 0,1 0,6 | 0,14 0,68 | 0,14 1,6 | 0,14 1,8 | 0,14 1,8 | 0,14 1,8 | 0,3 2,4 | 0,3 4 | 0,3 4 | 0,3 4 | 0,3 4 |
| Spring constant (Ncm/°) | 0,00206 |  |  |  |  | 0,0065 |  |  |  |  | 0,016 |  |  |  |  |

| G DA X ... X20 B21 (clockwise) B25 (anti-clockwise) | 060 |  |  |  |  | 075 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 95 +3 |  |  |  |  | 95 +3 |  |  |  |  |
| Operating mode | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % |
| Self-aligning torque (Ncm) min. of the spring MR max. | 0,56 0,6 | 0,56 1,0 | 0,56 2,0 | 0,56 4,0 | 0,56 5,6 | 1,1 1,8 | 1,1 5,0 | 1,1 8,4 | 1,1 11 | 1,1 11 |
| Spring constant (Ncm/°) | 0,026 |  |  |  |  | 0,05 |  |  |  |  |

| G DA Y ... X20 B21 (clockwise) B25 (anti-clockwise) | 060 |  |  |  |  | 075 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 65 +3 |  |  |  |  | 65 +3 |  |  |  |  |
| Operating mode | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % |
| Self-aligning torque (Ncm) min. of the spring MR max. | 0,56 1,5 | 0,56 4,0 | 0,56 6,2 | 0,56 6,5 | 0,56 6,5 | 1,1 4,65 | 1,1 11,7 | 1,1 12,5 | 1,1 12,5 | 1,1 12,5 |
| Spring constant (Ncm/°) | 0,026 |  |  |  |  | 0,05 |  |  |  |  |

| G DA Y ... X20 B23 (clockwise) B27 (anti-clockwise) | 060 |  |  |  |  | 075 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rotation angle (°) | 35 +3 |  |  |  |  | 35 +3 |  |  |  |  |
| Operating mode | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % | S1 100 % 40 % 25 % 15 % | S3 | S3 | S3 | S3 5 % |
| Self-aligning torque (Ncm) min. of the spring MR max. | 0,56 4,8 | 0,56 7 | 0,56 7 | 0,56 7 | 0,56 7 | 1,1 14 | 1,1 14 | 1,1 14 | 1,1 14 | 1,1 14 |
| Spring constant (Ncm/°) | 0,026 |  |  |  |  | 0,05 |  |  |  |  |

By edition of the present list, all former unit lists lose their validity especially. Illustrations without guarantee – modifications and supply availability reserved

4

l4

l6

d1

d2

t

1)

l3 l2

| G DA X and G DA Y |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 025 | 035 | 050 | 060 | 075 |
| Dim. | Dimensions in mm |  |  |  |  |
| a | --- | 10 | 11 | 11 | 18 |
| d1 | 25 | 35 | 50 | 60 | 75 |
| d2 | 3 | 4 | 6 | 8 | 10 |
| d3 | M3 | M3 | M4 | M4 | M5 |
| d4 | 26,2 | 36,4 | 51,6 | 61,6 | 76,2 |
| d5 | 11,4 | 14,6 | 20,8 | 24 | 29,5 |
| e | 18 | 25 | 35 | 40 | 50 |
| l1 | 55 | 47,5 | 65 | 78 | 93 |
| l2 | 25 | 27,5 | 35 | 42 | 53 |
| l3 | 15 | 10 | 15 | 18 | 20 |
| l4 | 100 | 100 | 150 | 150 | 200 |
| l5 | 3,2 | 4,5 | 4,7 | 6 | 8,5 |
| l6 | 5,5 | 7 | 8,5 | 10 | 12,5 |
| t | 3 | 3,5 | 5 | 6 | 8 |

l1

Sense of rotation  a

t

d2

e

d3

Fig. 4: Type G DA X 035 to 050 and G DA Y 025 to 050 (fastening holes: 2 x 180°)

d2

l3

t

l2

l1

l6

We ask you to please do not exceed the screw

l4

Sense of rotation  a

t

d2

d3

e

30°

d1

depth, this could lead to damage of the coil. The position of the shaft flattening (support of the  return spring) to the fastening threads is variable.

Fig. 5: Type G DA X 060 to 075 and G DA Y 060 to 075 (fastening holes: 3 x 120°)

d3

a

e

d4

1) Spanner flat for hooking the return spring with reversal of  rotation direction

Angular position to the fixing holes optional

d4

d2

l5

l5

t

l6

d5

l1

l2

l6

t

d2

t

d5

t

l2

l1

1) 1)

l4

d1

l3

d2

Sense of rotation

l4

d2

d1

l3

clockwise, ...B21, ...B23 anti-clockwise, ...B25, ...B27

Fig. 6: Type G DA X 035 to 075 and G DA Y 025 to 075

(with return spring)

By edition of the present list, all former unit lists lose their validity especially. Illustrations without guarantee – modifications and supply availability reserved

5

Safety

Please make sure that the described devices are suitable for your  application. Supplementary information concerning its proper  installation can be taken also from the Technical Explanations,  the effective DIN VDE0580 as well as the relevant specifications.

Installation instructions

The rotary solenoids may be inserted in any mounting position.  In the interest of service life and function of the bearing, please  make sure that impacts and bigger pressures on the rotation axis  in axial direction are avoided.

In case of vertical installation, the support of installed masses  should take place outside the solenoid. Furthermore it is advisable  to do not intercept bigger, with the axis connected masses with  the stops inside the solenoid but by external stops or damping  elements installed by the customer.

The device may not show any mechanical or electrical damages. For application with reduced rotation angle the range from end  position 0° (energized condition) should be used in order to achieve  the max. possible torque.

Standard devices will be delivered with free lead ends. Types with  clamps or connector on request.

For connection via plug connector Z KB X resp. Z KB G please pay  attention to max. continuous current of the connector.

Type code

Information and remarks concerning European directives  can be taken from the correspondent information sheet which is  available under Produktinfo.Magnet-Schultz.com.

Please make sure that the described devices are suitable for  your application. Our offers for these devices are based on  the assumption of maximal 8 in an FMEA severity table, i. e.  in case of malfunction of the device model as offered, there  is, amongst others, no jeopardy to life or limb. Supplementary  information concerning its proper installation can be taken  also from the –Technical Explanation, the effective  DIN VDE0580 as well as the relevant specifications.

This part list is a document for technically qualified personnel. This publication is for information purposes only and is not to be  regarded as a binding representation of the products, unless this  is expressly confirmed by us.

| Example G DA X |  | 050 | X20 | B21 |
| --- | --- | --- | --- | --- |
| Type | G DA X: 95° | 025 035 050 060 075 | X20 | Rotation angle 95° (GDAX size 035, 050, 060, 075) B01 without spring B21* with return spring clockwise B25* with return spring anti-clockwise Rotation angle 65° (GDAY all sizes) B01 without spring B21 with return spring clockwise B25 with return spring anti-clockwise Rotation angle 35° (GDAY all sizes) B03 without spring B23 with return spring clockwise B27 with return spring anti-clockwise |
|  | G DA Y: 65°, 35° |  |  |  |
| Size |  |  |  |  |
| Code for execution & protection class |  |  |  |  |
| Numbering |  |  |  |  |

2) only G DA Y available

3) G DA X size 035 in operating mode  S1/100% ED not available

Example

Type G DA X 060 X20 B01 Voltage 24 V DC Operating mode S1 (100 %)

Specials designs

Please do not hesitate to ask for our assistance with the solution  of your application-oriented task. In order to find rapidly a  reliable solution we need complete details about your application  conditions. The details should be specified as precisely as possible  in accordance with the relevant -Technical Explanations.

If necessary, please request the support of our corresponding  technical office.

Subject to Sales Terms applay, available at www.Magnet-Schultz.com 6

012025/15 Alt