Fibre-optic cable SOOC-DS-M6-2-R25

Part number: 552836

Data sheet

Feature Value

Note on materials RoHS-compliant

| Measurement method | Diffuse sensor |
| --- | --- |
| Min. object diameter | 0.1 mm |
| Fibre-optic - special feature | Standard |
| Operating distance | 140 mm |
| Ambient temperature | -55 °C...70 °C |
| Head size | M6 |
| Fibre-optic length | 2 m |
| Min. bending radius | 25 mm |
| Max. tightening torque | 3 Nm |
| Outside diameter | 2.2 mm |
| Material housing | Copper alloy PVC Nickel-plated |
| Material bre-optic | PE |
| Degree of protection | IP66 |
| Corrosion resistance class CRC | 1 - Low corrosion stress |

LABS (PWIS) conformity VDMA24364 zone III

Feb 13, 2025 – Subject to change - Festo SE & Co. KG