Retro-reective sensor SOOE-RG-R-PNLK-T

Part number: 8075667

Data sheet

Feature Value

Design Block design

| Conforms to standard | EN 60947-5-2 |
| --- | --- |
| Certication | RCM compliance mark c UL us - Listed (OL) |
| CE marking (see declaration of conformity) | As per EU EMC directive As per EU RoHS directive |
| UKCA marking (see declaration of conformity) | To UK instructions for EMC To UK RoHS instructions |
| Certicate issuing authority | UL E232949 |
| Note on materials | RoHS-compliant |
| Measuring principle | Optoelectronic |
| Detection method | Reection light barrier for transparent objects |
| Type of light | Red LED |
| Max. light spot | 300 mm at 3500 mm |
| Working range | 0 mm...3500 mm |
| Ambient temperature | -20 °C...60 °C |
| Reference material | Reference reector |
| Switching output | Push-pull |
| Switching element function | Switchable PNP, dark-switching NPN, bright-switching |
| Max. switching frequency | 500 Hz |
| Max. output current | 100 mA |
| Voltage drop | 0 V...1.5 V |
| Timer function | Via IO-Link® |
| Short-circuit protection | Pulsed |
| Protocol | IO-Link® |
| IO-Link®, protocol version | Device V 1.1 |
| IO-Link®, communication mode | COM2 (38,4 kBd) |
| IO-Link®, SIO mode support | Yes |
| IO-Link®, port class | A |
| IO-Link®, process data width OUT | 2 bit |

Feb 13, 2025 – Subject to change - Festo SE & Co. KG

Feature Value

IO-Link®, process data content OUT 1 bit (emitter disable)

1 bit (hold)

| IO-Link®, process data width IN | 2 bit |
| --- | --- |
| IO-Link®, process data content IN | 1 bit (stability alarm) 1 bit SSC (switching signal) |
| IO-Link®, minimum cycle time | 2.3 ms |
| IO-Link®, data memory required | 2 KB |
| DC operating voltage range | 10 V...30 V |
| Residual ripple | 10 % |
| Idle current | 25 mA |
| Reverse polarity protection | for all electrical connections |
| Electrical connection 1, connection type | Plug |
| Electrical connection 1, connection technology | M8x1 A-coded as per EN 61076-2-104 |
| Electrical connection 1, number of pins/wires | 3 |
| Electrical connection 1, type of mounting | Screw-type lock |
| Material of pin contacts | Brass, gold-plated |
| Type of mounting | With through-hole for M3 screw |
| Tightening torque | 0.8 Nm |
| Mounting position | Any |
| Product weight | 10 g |
| Housing material | PC PMMA |
| Ready status indication | LED green |
| Switching status indication | LED yellow |
| Function reserve indication | LED yellow, ashing |
| Setting options | IO-Link® Potentiometer Teach-in |
| Degree of protection | IP65 IP67 IP69K |
| Insulation voltage | 500 V |
| Surge resistance | 1 kV |
| Corrosion resistance class (CRC) | 1 - Low corrosion stress |
| LABS (PWIS) conformity | VDMA24364 zone III |

Contamination level 3

Feb 13, 2025 – Subject to change - Festo SE & Co. KG