Fork light barrier SOOF-P-FL-ST-C50-P

Part number: 553563

Data sheet

Feature Value

Approval RCM trademark

c UL us listed (OL)

| CE mark (see declaration of conformity) | To EU EMC Directive In accordance with EU RoHS Directive |
| --- | --- |
| UKCA marking (see declaration of conformity) | To UK instructions for EMC To UK RoHS instructions |
| Note on materials | RoHS-compliant |
| Measured variable | Position |
| Measuring principle | Optoelectronic |
| Measurement method | Fork light barrier |
| Type of light | Red |
| Min. object diameter | 0.3 mm |
| Ambient temperature | -10 °C...60 °C |
| Repetition accuracy | 0.03 mm |
| Switching output | PNP |
| Switching element function | Switchable |
| Hysteresis | 0.25 mm |
| Max. switching frequency | 2000 Hz |
| Max. output current | 100 mA |
| Short circuit current rating | Pulsed |
| Operational voltage range DC | 10 V...30 V |
| Residual ripple | 10 % |
| No-load supply current | 30 mA |
| Reverse polarity protection | For operating voltage connections |
| Electrical connection | 3-pin M8x1 Plugs |
| Size | Fork 50x55 mm |
| Fork gap | 50 mm |
| Product weight | 30 g |
| Material housing | PC |
| Switching status indication | Yellow LED |
| Setting options | Teach-in |

Feb 13, 2025 – Subject to change - Festo SE & Co. KG

Feature Value

Degree of protection IP67

| Insulation voltage | 500 V |
| --- | --- |
| Immunity to surge | 0.8 kV |
| Corrosion resistance class CRC | 0 - No corrosion stress |
| LABS (PWIS) conformity | VDMA24364 zone III |

Pollution degree 3

Feb 13, 2025 – Subject to change - Festo SE & Co. KG